ROR DX195 DISC BRAKE INSPECTION

1.	Release trailer brakes.

2.	Rattle chamber and check slide pins are free.

3.	Back brake off.

4.	Remove brake pads.

5.	Remove clevis pin from actuator.

6.	Wind brake back on 2 turns.

7.	Operate lever 10 to 15 times.  It should operate smoothly.  If it 	feels like it has a ratchet action then the mechanism is partly 	seized.

8.	If the operation is OK – grease through the adjuster cap with low 	pressure grease gun:

Remove a top bolt by mechanism – when grease starts to come out 	of the holes, its full.

9.	Remove the metal caps (on the ends of the slides) lubricate slide 	pins with WD40, and put a light smear of grease on inside of end 	cap and refit.

10.	Stamp the caliper with a ‘G’ to show its been greased.  (They will 	not require greasing again).